Delphi HASP components

version 1.0

Copyright © Serge Sushko, 1997-98

New versions available: http://members.tripod.com/~sushko/

e-mail comments to: sushko@iname.com.

Delphi HASP components is a set of native Delphi components used to read/write data from/to HASP e-keys by Aladdin Knowledge Systems, Ltd in order to make a proper software protection against software piracy. Designed as wrapper for native HASP API, Delphi HASP components provide easy-to-use developer interface to Aladdin's keys and extended functionality in encoding data, smart caching mechanism, international language support and some other advanced features.

In version 1.0 Delphi HASP component package consist of the following components:

THASP3 is a basic HASP component (derived from TComponent), wrapping functionality of HASP3 keys

TMemoHASP wrappes functionality of MemoHASP keys

TTimeHASP provides interface to TimeHASP key family.

The following datatypes are declared:

THASPType

EHASPError

Coming soon:

TFAS component, implementation of Aladdin's Full Autorization System.

Installation

Before using the Delphi HASP Components under Windows95 or WindowsNT you need to install the HASP driver,  provided by Aladdin Knowledge Systems, Ltd. You can find the required software on your HASP Developer Kit CD, on my homepage at

http://members.tripod.com/~sushko/hasp/hinstall.zip

or on one of the Aladdin's Internet sites:

http://www.aladdin.co.il/

http://www.aladdin.ru/

http://www.aks.com/

http://www.hasp.com/

The driver installation file is named HINSTALL.EXE and contains drivers for all 32-bit operation systems by Microsoft Corp.

Delphi2 installation.

- Close all Delphi applications

- Unzip files from the HASP.ZIP to any directory (C:\HASP, for example)

- Put the contents of the \HASP\Delphi2\ to your \Delphi\Lib directory

- Move HASP.KWF file from the \HASP\Help to your \Delphi\Bin directory

- Copy HASP.CNT and HASP.HLP files from the \HASP\Help to your \Delphi\Bin

- Copy HASP.CNT and HASP.HLP files from the \HASP\Help to \HASP\Demo directory     (the help files are used by the Delphi IDE and the demo project both)

- Start your \Delphi\Help\Tools\helpins.exe programm

- Choose File|Open and select the \Delphi\Bin\Delphi.hdx file

- Choose File|Add and select the \Delphi\Bin\hasp.kwf

- Choose File|Save, wait for the help system compilation will be completed and exit the programm

- Start Delphi

- Select Component|Install Component and press Add button

- Locate \Delphi\Lib\hasp.dcu file and press Open button;

- Press Ok button

- Wait for library rebuild will complete

- Choose File|Open and open the \HASP\Demo\demo.dpr file

- Choose Run|Run to run the demo project

Delphi3 installation.

- Close all Delphi applications

- Unzip files from the HASP.ZIP to any directory (C:\HASP, for example)

- Put the contents of the \HASP\Delphi3\ to your \Delphi\Lib directory

- Copy HASP.CNT and HASP.HLP files from the \HASP\Help to your \Delphi\Bin

- Copy HASP.CNT and HASP.HLP files from the \HASP\Help to \HASP\Demo directory (the help files are used by the Delphi IDE and the demo project both)

- Add line ":Link hasp.hlp" to the file \Delphi\Help\Delphi.cfg

- Add line ":Include DELPHI3.CFG" to the \Delphi\Help\hasp.hlp

- Start Delphi

- Select Component|Install Packages and press Add button

- Locate \Delphi\Lib\hasp.dpl file and press Open button;

- Press Ok button

- Choose File|Open and open the \HASP\Demo\demo.dpr file

- Choose Run|Run to run the demo project

THASPType type

The THASPType type defines the key types for Delphi HASP components.

unit

THASP

type

THASPType = (htUnknown, htHASP1, htHASP4, htHASP3, htTimeHASP, htTimeHASP4)

Description:

The THASPType defines the possible key types of the HASP component. It is the type of the HASPType property. The following table lists the possible values:

Value		Meaning

--------------------------------------------------------------

htUnknown	Unrecognized

htHASP1		HASP1

htHASP4		HASP4

htHASP3		HASP3

htTimeHASP	TimeHASP

htTimeHASP4	TimeHASP4

EHASPError

The EHASPError type defines the HASP key specific type of exception.

unit

THASP

type

EHASPError = class(Exception);

Description:

The EHASPError is used to identify the exception raised by Delphi HASP components.

THASP3

THASP3 is a component providing access to HASP3 key family

Type

THASP3=class(TComponent);

Unit

THASP

Description

Use THASP3 to get access to HASP3 electronic keys by Aladdin, Co. Place a component on a form and set HighPassword and LowPassword to the values, received from Aladdin with your HASP key series. Use Cached property and Refresh method to manage the component's caching mechanism. Set Port property to the propper value (represented by port address) or leave the default value to make the component search the key on all present ports. Use the Connected property to determine whether the key was found on the specified port and HASPType property to determine the type of the key connected to the port.

Use the Open method before accessing the key and Close method after you have finished using it. In special cases use the f and GetHASPCode functions to retrieve the key-specific integer values. To encode application-specific data use EncodeInteger and EncodeString functions. On error use the ErrorMessage function to retrieve the error description.

property HighPassword: Integer;

Use HighPassword to set the first (from two) password to access the HASP key of your series. If HighPassword is set incorrectly, thecomponent will return you default (illegal) values as result of function calls.

property LowPassword: Integer;

Use LowPassword to set the second (from two) password to access the HASP key of your series. If LowPassword is set incorrectly, the component will return you default (illegal) values as result of function calls.

Note.

It is a good idea to store wrong numbers in HighPassword and LowPassword properties at design-time and set them to propper values at run-time only. The reason is that the values written to component properties at design-time are stored in DFM file (and in EXe or DLL after compilation) and thus could be easily retrieved by a hacker.

procedure Open;

Use this before first call to the THASP component's method or property. The Open method is equivalent of setting Cached property to False if initially it was set to True or calling Refresh method with Cached property set to True. Anyway, the call of Open methos sets all the properties of the component to real values of the key if LowPassword and HighPassword are set to correct values.

property HASPType : THASPType;

Use HASPType property to determine the type of the key.

property Cached: Boolean;

The Cached property (combined with Refresh method) is used to manipulate the HASP component's caching mechanism. The problem is that the HASP key driver in some cases could be rather slow. This may happen if there is no HASP key attached to the computer (and you want your software to work in demo mode) or under some national releases of MS Windows'95 (Win95 OSR2 Russian, for example). The timeout between the API call and it's responce can take 1-2 seconds, and, called in a cicle (in TDataSource.OnDataChange event, for example) can significantly drop perfomance of your application. The only way to eliminate this effect in such cases is to store results of HASP API calls in local variables of the THASP3 component and it's descenders and use the cached values instead of real API call results. To refresh the cach contents use the THASP3.Refresh method.

If the Cached property of the component is set to True, values of the following properties will be cached:

For THASP3:

Port

HASPType

Connected

For TMemoHASP:

Port

HASPType

Connected

HASPID

For TTimeHASP:

Port

HASPType

Connected

HASPID

procedure Refresh;

The Refresh method (combined with Cached property) is used to manipulate the HASP component's caching mechanism. The problem is that the HASP key driver in some cases could be rather slow. This may happen if there is no HASP key attached to the computer (and you want your software to work in demo mode) or under some national releases of MS Windows'95 (Win95 OSR2 Russian, for example). The timeout between the API call and it's responce can take 1-2 seconds, and, called in a cicle (in TDataSource.OnDataChange event, for example) can significantly drop perfomance of your pplication. The only way to eliminate this effect in such cases is to store results of HASP API calls in local variables of the THASP3 component and it's descenders and use the cached values instead of real API call results. To refresh the cach contents use the THASP3.Refresh method.

property Port : Integer;

The Port property determines the LPT port address the HASP key is attached to. Set the Port property to 0 to invoke the THASP3 component to determine the port number itself (default). Set the Port property to 102 to tell the HASP3 component to search the HASP key on LPT1 port. Refer to HASP documentation to get values for other port addresses.

In common cases you do not need to set the Port property to value other from 0 (default); the Open or Refresh methods will automatically set the proper value of the property. Manual use of the Port property can be useful if there is more than one HASP key attached to the computer or the specific hardware attached to one of the LPT ports conflicts with HASP driver by Aladdin, Co.

property Connected : Boolean;

Use the Connected property to determine whether the HASP key is connected to the port, specified in the Port property. Be carefull with Connected property if the Cached property of the component is set to true.

procedure GetHASPCode(iSeedCode : Integer; var i1, i2, i3, i4 : Integer);

The GetHASPCode function implements the standard HASP API function GET_HASP_CODE, retreiving four unique integer values i1, i2, i3, i4 from the HASP key. The values retrieved depend on the key (i.e. hardware) itself and the iSeedCode parameter. In other words, you will get the same integer return values twice only if you will use the same hardware key and call this function with the same iSeedCode parameter. This return values of this function can be used in order to determine that the key attached to the computer is the same that was attached the last time the function was called, for data encoding and so on.

If there is no proper key attached to the computer, the passed in values of i1, i2, i3, i4 parameters will be set to zero by wrapped HASP API function. Use GetHASPCodeEx method instead of calling GetHASPCode function to get random values in i1, i2, i3, i4 parameters if there is no key attached to the computer or if illegal passwords are provided for the existing key. Use EncodeString and EncodeInteger methods to implement the simple XOR encoding based on the GetHASPCode and GetHASPCodeEx return values.

procedure GetHASPCodeEx(iSeedCode : Integer; var i1, i2, i3, i4 : Integer);

The GetHASPCodeEx function extends functionality of the standard HASP API function GET_HASP_CODE, retreiving four unique integer values i1, i2, i3, i4 from the HASP key. and wrapped by GetHASPCode method of the component The values retrieved depends on the key (i.e. hardware) itself and the iSeedCode parameter. Return values of these functions can be used in order to determine that the key attached to the computer is the same that was attached the last time the function was called, for data encoding and so on.

If there is no proper key attached to the computer, the function (unlike the GetHASPCode method) will return random values for passed in i1, i2, i3, i4 parameters, what can me usefull, for ixample, in encoding purpuses. Use EncodeString and EncodeInteger methods to implement the simple XOR encoding based on the GetHASPCodeEx return values.

function EncodeInteger(iSeedCode, iValue : Integer) : Integer;

Use the EncodeInteger function to implement a simple encoding based on the GetHASPCode method return values. The function encodes an integer value passed in iValue parameter by the XOR algorythm, using the four integer key-specific values, returned by the GetHASPCode function with passed-in iSeedCode parameter, and returns the result of the encoding. To decode the encoded value, call EncodeInteger one more time, using the same iSeedCode parameter and the same hardware key. According to the GetHASPCode function uniqueness for the given hardware key, the decoding of the encoded value can be performed only by those who owns the same hardware key and know the iSeedCode value used for the encoding.

Attention!

The EncodeInteger and EncodeString are very simple algorythms and can be used only to protect data from non-professional unauthorized access. Never use them to encode important information such as credir card numbers! To implement professional-quality encoding please use proper algorythms such as PGP, DES, etc.

function EncodeString(iSeedCode, sValue : String) : String;

Use the EncodeString function to implement a simple encoding based on the GetHASPCode method return values. The function encodes a string value passed in sValue parameter by the XOR algorythm, using the four integer key-specific values, returned by the GetHASPCode function with passed-in iSeedCode parameter, and returns the result of the encoding. To decode the encoded value, call EncodeString one more time, using the same iSeedCode parameter and the same hardware key. According to the GetHASPCode function uniqueness for the given hardware key, the decoding of the encoded value can be performed only by those who owns the same hardware key and know the iSeedCode value used for the encoding.

Attention!

The EncodeInteger and EncodeString are very simple algorythms and can be used only to protect data from non-professional unauthorized access. Never use them to encode important information such as credir card numbers! To implement professional-quality encoding please use proper algorythms such as PGP, DES, etc.

procedure Close;

Use this method after you have finished working with the component, for example -- before terminating the application or before user will change the key (i.e. hardware) attached to the computer. The Close method does nothing special except setting all the properties of the component to the default values. There is no special need of calling this method during the programm work.

function ErrorMessage(iErrorCode : Integer) : String;

Use this function to retrieve the text description of the error to use it, for example, in an exception raised by your application. The iErrorCode parameter takes the values of the HASP API error codes (see your HASP API documentation for further details). The message returned is language-specific and, as far as Delphi HASP components support international languages, you can retrieve the proper language file from the site, described in the About topic.

TMemoHASP

TMemoHASP is a component providing access to MemoHASP key family

Unit

THASP

type

TMemoHASP = class(THASP3)

TMemoHASP is a direct descendant of the THASP3 component and used to get access to MemoHASP electronic keys by Aladdin, Co. Used to communicate with memory-enabled keys, TMemoHASP has specific memory-access functions such as ReadWord/WriteWord and ReadBlock/WriteBlock. To determine the memory size of the key (dependent of the key type and stored in the HASPType property), use the MemorySize property. All the MemoHASP keys also has unique identifiers, which can be obtained via HASPID property.

To access TimeHASP keys, use TTimeHASP components instead.

property HASPID: Integer;

All the Aladdin's HASP keys except HASP3 provide an unique key-specific number, which can be obtained by the application via the HASPID property. You may use this number if you want your application to work with one exact key, or to add a registration number (calculated from key number), or in some other cases. Be carefull with HASPID property if the Cached property of the component is set to true.

property MemorySize : Integer;

Use the MemorySize property to determine the size of MemoHASP memory buffer. The memory size depends on the type of the key (stored in the HASPType property)

procedure ReadBlock(iStartAddress: LongInt; pBlock : Pointer; iBlockLen : LongInt);

Use the ReadBlock method to read iBlockLen bytes from the MemoHASP memory, stored there from the iStartAddress address. The data can be written to memory by WriteWord and WriteBlock methods. The Pointer parameter must point to the memory block with at least iBlockLen bytes allocated.

procedure ReadWord(iOffset : LongInt; iWord : LongInt);

Use the ReadWord method to read a word (four-byte integer value) from the MemoHASP memory, stored there from the iOffset address. The data can be written to memory by WriteWord and WriteBlock methods.

function ReadString(iStartAddress: LongInt; iStrLen : LongInt) : String;

Use the ReadString method to read a string iBlockLen bytes in length from the MemoHASP memory, stored there from the iStartAddress address. The data can be written to memory by WriteWord, WriteBlock and WriteString methods.

procedure WriteBlock(iStartAddress: LongInt; pBlock : Pointer; iBlockLen : LongInt);

Use the WriteBlock method to write iBlockLen bytes to the MemoHASP memory, storing them from the iStartAddress address. The Pointer parameter points to the data writing and must have at least iBlockLen bytes allocated.    The memory can be read later by ReadWord and ReadBlock methods.

procedure WriteWord(iOffset : LongInt; iWord : LongInt);

Use the WriteWord method to write a word (four-byte integer value) to the MemoHASP memory. The memory can be read later by ReadWord and ReadBlock methods.

procedure WriteString(iStartAddress: LongInt; sString : String);

Use the WriteString method to write a sString string to the MemoHASP memory, storing them from the iStartAddress address. The memory can be read later by ReadWord, ReadBlock and ReadString methods.

TTimeHASP

TTimeHASP is a component providing access to TimeHASP key family

Unit

THASP

Type

TTimeHASP = class(TMemoHASP);

TTimeHASP is a direct descendant of the TMemoHASP component and used to get access to TimeHASP electronic keys by Aladdin, Co. Used to communicate with timer-enabled keys, TMemoHASP has specific date and time-access properties Date and Time. All the TimeHASP keys also has unique identifiers, which can be obtained via HASPID property

All the TimeHASP keys can be divided into two groups: memory-enabled and not memory enabled. The memory-enabled TimeHASP keys have up to 512 bytes of memory on board, which can be used to store application-specific data. As far as TTimeHASP is a direct descendant of TMemoHASP component, you can use memory-managing TMemoHASP methods ReadBlock/WriteBlock and ReadWord/WriteWord with such hardware keys. Other TimeHASP keys have no memory on board and only THASP3 methods are available for these keys (memory method call will raise an exception).

To determine what types of TimeHASP keys contain memory ob board, refer your HASP documentation. To find out what kind of the key is attached to the computer just now use HASPType property. Be sure to Refresh the TTimeHASP cach before reading HASPType property if Cached property is set to True.

property Date : TDate;

Use the Date property to read or write date value to the TimeHASP key clock.

property Time : TTime;

Use the Time property to read or write time value to the TimeHASP key clock.

function GetDateTime : TDateTime;

The GetDateTime function is used to read both date and time values from the TimeHASP clock.

procedure SetDateTime(ADateTime : TDateTime);

The SetDateTime procedure is used to set both date and time values to the TimeHASP clock.

Full Authorization System (FAS)

Aladdin's Full Authorisation System (FAS) is a standard (a set of rules) of storing protection information in the HASP key memory if the key has memory on board. FAS system stores application launch count for the MemoHASP keys and application expire date for the TimeHASP keys. The number of applications that could be protected by the FAS and a single key depends on the key's memory size and vary from eight to sixteen.

All the Aladdin's protection systems (such as Envelope) use the same rules of protection data storage. To implement a FAS-compatible protection in a Delphi component we must know the rules of storing data in different keys. This is not a big secret (as far as the hacker won't be able to access the key memory if he doesn't know passwords of your key series), but for most protection systems there is no need to know that rules and it was rather difficult to retrieve them from the Aladdin's office. While experimenting with the Aladdin's software I've found out the MemoHASP memory map and location of FAS information in it, but I still do not know how this data is stored in the TimeHASP key. When this information will become available to me I'll release a new version of a Delphi HASP component package with the FAS component: all the required functions are ready and the only problem is the TimeHASP memory map... Be close to news on my homepage.

Thank you for your interest in Delphi HASP Components.

Serge Sushko

sushko@iname.com

http://members.tripod.com/~sushko/